SECTION 12495

SUN CONTROL PANELS

*****  This section can be used to specify fixed, rigid panels with open weave fabric installed over windows, sloped glazing, and skylights to provide sunlight filtration.

For Draper sun control panels there are options for size, fabric, method of installation, frame, and frame finish.  Refer to Draper product literature for availability of options and selection guidelines.  Various options are listed in this specification section and the specifier will need to select the appropriate options, verify their availability, and delete non-applicable ones.

Operable window shades can be specified in SECTION 12492 - MANUALLY OPERATED WINDOW SHADES, SECTION 12493 - MOTORIZED WINDOW SHADES, SECTION 12494 - OPAQUE WINDOW SHADE SYSTEM, SECTION 12496 - SKYLIGHT SHADES, SECTION 12497 - DUAL ROLLER WINDOW SHADES, AND SECTION 12498 - VERTICAL FOLDING WINDOW SHADES.  *****

PART 1 - GENERAL

1.1	SUMMARY

A.	Section includes:  Fixed panels with open weave fabric installed over [windows] [sloped glazing] [skylights] to provide sunlight filtration.

B.	Related sections:

*****  List other specification sections dealing with work directly related to this section such as the following.  *****

1.	Section 08500 - Windows:  [__________] windows to receive sun control panels.

2.	Section 08600 - Skylights:  Skylights to receive sun control panels.

3.	Section 08960 - Sloped Glazing Assemblies:  Sloped glazing to receive sun control panels.

1.2	SUBMITTALS

A.	Submit in accordance with Section 01330 - Submittal Procedures:

1.	List of proposed products and product data.

2.	Shop drawings showing dimensions and attachment method.

3.	Samples [for selection by Architect]:

a.	Fabrics.

b.	Metal finishes.

4.	Sun Control Panel Schedule listing rooms, field verified opening dimensions, quantities, type of panel, fabric, and color.

5.	Manufacturer's installation and maintenance instructions.

1.3	DELIVERY, STORAGE, AND HANDLING

A.	Do not deliver sun control panels until building is enclosed and construction within spaces where panels will be installed is substantially complete.

B.	Deliver products in manufacturer's original, unopened, undamaged containers with labels intact.

C.	Label containers and panels according to Sun Control Panel Schedule.

PARTS 2 - PRODUCTS

2.1	ACCEPTABLE MANUFACTURERS

A.	Draper, Inc., 411 South Pearl Street, Spiceland, Indiana 47385-0425; 765-987-7999.

B.	Manufacturers of equivalent products submitted and approved in accordance with Section 01630 - Product Substitution Procedures.

2.2	SUN CONTROL WINDOW PANELS

*****  Select type of frame, fabric, and method of installation from the following options.  *****

A.	Type:  Fixed, rigid aluminum framed panels with open weave fabric to be installed on [interior] [exterior] of [windows] [sloped glazing] [skylights]; FlexShade Panels as manufactured by Draper, Inc.

*****  Maximum size for FlexShade Panels is 60 by 120 inches (1524 mm by 3048 mm).  *****

B.	Panel size:  [[_____] by [_____] [inches] [mm].] [As indicated on Drawings.]

C.	Framing:

1.	Panel framing:  Extruded aluminum perimeter framing.  Dimensions, wall thickness, and material selected by manufacturer to accommodate panel size and type of installation.  Minimum frame size of [3/4 by 7/16 inch] [19 by 11 mm].

2.	Provide center cross braces for panels exceeding [40 inches] [1016 mm] in length or width.

*****  FlexShade panels can be installed on exterior side of windows with mounting channels and keeper track.  For exterior installation include the following paragraph.  *****

3.	Exterior mounting track:  Provide extruded aluminum flanged side mounting channel [, intermediate mounting channels,] and top and bottom keepers for mounting sun control panels on exterior side of windows.  Provide screws of sizes recommended by manufacturer for type of installation.  Screw heads to match finish of frame.

*****  For interior installation include the following paragraph.  *****

3.	Attachment clips:  Wing and screws of sizes recommended by manufacturer for type of installation.  Clips designed for easy removal of panels for maintenance and cleaning.  Clips and screw heads to match finish of frame.

4.	Finish:  [Mill finished aluminum] [Dark bronze anodized] [White baked enamel].

D.	Mounting spline:  Vinyl compressible spline factory inserted in frame groove to secure fabric.

2.3	FABRIC

*****  Draper provides 11 fabrics for fixed sun control panels.

SheerWeave Series SW1000

SheerWeave Series SW3000

SheerWeave Series SW2000

SheerWeave Series SW2100

ShearWeave Series SW2900

ShearWeave Series SW2600

SheerWeave Series PW3500

SheerWeave Series PW4100

SheerWeave Series PW4400

E Screen

M Screen

Refer to Draper product literature for available shade fabrics and selection guidelines.  *****

A.	Material:  [Woven vinyl-coated fiberglass screening; SheerWeave Series SW1000] [Vinyl coated fiberglass and vinyl coated polyester woven into 63 x 15 mesh; SheerWeave Series SW3000]   [500 denier fiberglass, vinyl coated and woven into a 2 x 2 basketweave; SheerWeave Series [SW2000] [SW2100] [SW2900] [SW2600]]  [Vinyl coated polyester yarn woven into basketweave pattern; SheerWeave Series [PW3500] [PW4100] [PW4400]]  [PVC coated fiberglass yarn woven in 2 x 2 basketweave; E Screen] [PVC coated fiberglass in 1 by 2 weave; M Screen]as provided by Draper, Inc.

*****  Refer to Draper product literature for attributes of selected fabric.  *****

B.	Attributes:

1.	Weight:  [_____] ounces per SY.

2.	Thickness:  [_____] inch.

3.	Roll width:  [_____] inches.

4.	Average open area:  [_____] percent.

5.	[Fire retardant,] [washable,] [stain resistant,] [anti-static treated,] [__________].

*****  Refer to Draper fabric samples for available fabric colors and patterns.  *****

C.	Color and pattern:  [[__________], No. [_____].]  [As indicated in Color Schedule on Drawings.]  [As selected by Architect from manufacturer's standard range.]

PART 3 - EXECUTION

3.1	PREPARATION

A.	Field verify [window] [sloped glazing] [skylight] dimensions prior to fabrication.

B.	Coordinate installation of panels with type and dimensions of [window] [skylight]  framing.

3.2	INSTALLATION

A.	Install sun control panels at locations indicated on Drawings and approved Sun Control Panel Schedule.

B.	Comply with panel manufacturer's written instructions.  Install with panel spline against opening.

C.	Install with self tapping screws or other fasteners of type, size, and spacing recommended by manufacturer.  Uniformly space fasteners.

*****  For exterior installation use the following paragraph.  *****

D.	Install mounting channels first.  Slide sun control panel in from lower end.  Install keeper tracks to secure panel in frame.

*****  For interior installation use the following paragraph.  *****

D.	Install with wing clips and screws.  Rotate wing clip over frame to secure panel.  Do not overtighten screws.

E.	Ensure panels are level, plumb, and in alignment with windows.

F.	Protect panels from damage from construction operations.  If damage occurs, remove and replace panels.

END OF SECTION