Project Name:

SECTION 12492

Horizontal Louver Blinds

# Part 1 - General

# DESCRIPTION

- 1/2" Horizontal Louver Blinds With Aluminum Slats.

  - SUBMITTALS

- Manufacturer’s complete CSI 3- part specification sheet.

- Product Sample: Submit working hand sample or mock up blind as required.

- Color Sample: Submit two 6" samples of aluminum slat indicating color and dimensions.

  - DELIVERY, STORAGE AND HANDLING

- Product to be delivered in manufacturer’s original packaging.

- Products to be handled and stored to prevent damage to materials, finishes and operating mechanisms. Store in a clean, dry area, laid flat to prevent sagging and twisting of packaging.

  - EXTRA STOCK

- Describe extra attic stock as required:

.

# Part 2 – Products

  - ACCEPTABLE MANUFACTURERS

- Springs Window Fashions or approved equivalent.

- Substitutions Request: Submit for approval under provisions of section 01600.

  - HORIZONTAL BLINDS

- Product: Bali S4000 Micro Blinds

- Color

Color Name:

Color Number:

- Headrail: Shall be 1" high by 1 1/2" wide, curved headrail face w/ light blocking lip at the lower back side, made of phosphate treated steel, finished with a polyester baked enamel paint finish and shall measure .025" thick.

- Bottomrail: Shall be completely enclosed tubular shape made of phosphate treated steel, finished with a polyester baked enamel paint finish and shall measure .025" thick

- Slats: 1/2" wide slats shall be aluminum alloyed for maximum strength and flexibility and will be treated with an anti-static and anti-microbial paint finish. Slats shall be .006" thick.

- Tilter shall me made of injection-molded thermoplastics for smooth low friction operation and will incorporate a clutch mechanism to prevent damage due to over tilting.

Options:

Tilt limiter will limit slat operation between a select range of operation. Select degrees up and degrees down from horizontal (up/down settings are nominally 0, 20, 40, 60 and 80):

degrees up and       degrees down from horizontal

- Cord lock shall be a snap in design incorporating a stainless steel wear guard over which cord pass. Floating shaft locking pin will lock blind at any position and shall be crash proof.	 End of lift cords will be treated with color coordinated plastic tassels.

Options:

Top lock cord lock provides for only two blind positions, fully down or fully raised.

Ring pull provides a single plated steel ring in lieu of tassels with a nominal 4" cord length.

- Braided ladder shall be made of 100% high tenacity polyester incorporating two extra strength rungs per ladder to support slats.

  - Fabrication

  - Prior to fabrication, verify actual opening dimensions by on-site measurement. Calculate blind dimensions to fit within specified tolerances.

  - Fabricate blinds to fill openings from head to sill and jamb-to-jamb. Locate blind divisions at mullions.

  - Fabricate blinds to fill all exterior window openings except at doors, door                                sidelights and transoms unless noted.

## Part 3 – Execution

  - Inspection

A.  Verify that the area in which the blinds are to be installed is free of    conditions that interfere with blind installation and operations. Begin blind installation only when unsatisfactory conditions have been corrected.

  - Installation

  - Install blinds in accordance with manufacturer’s instructions except as otherwise specified herein.

  - Install blinds with adequate clearance to permit smooth operation of the blinds.

  - Set tilt and lift controls. Demonstrate blinds to be in smooth, uniform working order.

  - Maintenance and Cleaning

  - Maintain and clean blinds in accordance with manufacturer’s instructions.

### End of Section